CS-6040

ASSIGNMENT 3

TECHNICAL REPORT

Name:  Gourav Sharma

Roll No: cs21M020

Performance of Queuing:

- NOQ

- CIOQ

- NOQ:

- NOQ means no queueing or no buffers are going to be used.

- CIOQ:

- Combined Input-Ouput Queueing. Both input and ouput buffers were used.

- IMPLEMENTATION:

- Both NOQ and CIOQ contained three phases.

- Traffic Generation

- Scheduling

- Tranmission

- A)Three threads were were created I.e. one for each phase.  In Traffice Generation and Tranmission, in turn nested threads were created. If

- There are n input and n output ports, then n internal threads were created in traffic generation and n internal threads in transmission phase.

- B) Scheduling was the core part. In NOQ, scheduling was direct.

- In CIOQ,  Parallel Iterative Matching was used, in order to make utilization maximum and minimize the output contention in each slot.

- The algorithm used for implementing was Maximum Bipartite Matching. As, if first L slots were considered then there were L*N

- Packets, out of which N should be transmitted in one slot.

- The backplane speed should be N times faster than the input line rate.

- So, from any input queue, L packets can go to any of the output queues, this can be stored as a graph.

- The graph data structure was implemented as Adjacency Matrix.

- E)So the goal was to maximize the matching. Matching means making

- the connections  in the graph such that no edges were adjacent.

- The PIM technique will maximize the efficiency as it would try to minimize the output contention.

- The space complexity and time complexity for PIM algorithm  is O(N^2)when number of input ports and output ports are equal to N.

- Now, as there are k servers in each output port i.e. there are k buffers in each output port. So, the input ports where there is output contention, we can forward the packets into the output queues whose servers or buffers are empty.

- RESULTS:

- Experiment reading proves that CIOQ performs much better than

- NOQ, as average output port utilization in NOQ is around 40% and

- CIOQ ‘s average output port utilization in CIOQ is around 75%.

- Average packet delay is fixed in NOQ which is 1 unit. Average packet delay in CIOQ is around slightly greater than the number of buffers in the input queue.

- For around L = K = 0.7*N, experimental analysis shows us that average port utilization is maximum,

- VISUALIZATION: